Visualization of Kinematics

Hyun Seok Shin

This document intents to explain the forward kinematics and visualization code.  All the details from modeling Darwin OP1, OP3, and human, for those not familiar with the Denavit-Hartenberg (DH) convention and kinematics tree structure, to forward kinematics and visualization algorithm are included so that the others can easily use and build upon it.

Table of Contents

- System modeling

- Forward kinematics

- Visualization

1. System modeling

Denavit-Hartenberg (DH) convention:

DH parameters provide a homogeneous transformation matrix to map from the previous frame to the current frame.  There are four DH parameters: , d, a, and .   is the angle between xn–1 and xn; dn and an are distances between joint n and n – 1 along zn–1 and xn, respectively; and  is angle between zn–1 and zn.

After DH parameters of a system are determined, make DH table as follow:

| Joint |  | d | a |  |
| --- | --- | --- | --- | --- |
| 1 |  | d1 | a1 |  |
| n |  | dn | an |  |

A DH homogeneous transformation matrix describes position of a joint and link orientation, which is defined as:

where T(1:3, 1:3) is the link orientation and T(1:3, 4) is the position of the link.  Using the matrix, the global position of a link i can be calculated as:

.

System of kinematic chains:

Darwin OP1, OP3 and human consist of multiple kinematic chains.  Use Parent and Child to model such system.  Starting from the parent branch, define its child.  Then the new child branches will become new parent and recursively use same pattern to model them.  This will be more evident looking at the examples.

Below is a case of the kinematic chains of ROBOTIS OP1 and OP3.

Fictitious branch is the parent branch and right leg, left leg, neck, right arm, and left arms are the child branches of the fictitious branch.  Since the humanoids do not have torso branch, the body mass is attached to the first joint of the right arm.

Below is a case of the kinematic chains of human.

In this case, there are grand child branches, thus torso becomes a succeeding parent branch.  The first parent branch is the fictitious branch and its child are right leg, left leg and torso branches.  Then, the torso becomes a new parent branch and it has child branches of neck, right arm and left arm.

ADOL convention of assigning DH parameters:

DH structure can be redundant.  To avoid, follow the ADOL convention when assigning DH frames to each joint.

- All z-axis in sagittal plane should be pointing to the right.

- Link length should be assigned to a as much as possible.

- There should not be any negative a value, as it is counterintuitive.

2. Forward kinematics

Structure of the code (forward kinematics)

The code is dived into two parts: forward kinematics and visualization.  It will first calculate forward kinematics then it will visualize the forward kinematics result.

In the main directory, open MAIN.m.  This is the main script for the forward kinematics and visualization.  In the directory, there is “Forward_Kinematics” directory that has all the functions used in the main script.  User should create new directory for a system (e.g. “Human_2D and Human_3D).  In the new directory, there should be the system input files, which are:

- DH structure (.txt),

- Link length (.txt), and

- Mass properties (.txt).

Joint angles are optional.  If joint angles are not provided, then code will simply calculate the home configuration.

- Joint angles (.xlsx) - note that c-code output files need to be converted to newer excel type (.xls  .xlsx).  Else the code cannot the file.

Note that above files should follow the ADOL convention.

To use the code, a user only needs define input files names listed below and a system name.

After defining the input files and the system, the code will read and sort the data using readMass.m, readLink.m, readDH.m, and readJV.m.  The functions can handle any structure as long as ADOL convention is followed.

Then, the code proceeds to forward kinematics calculation.

ForwardKinematics.m will calculate the forward kinematics.  It will output JK (joint positions), CK (each link’s CoM positions) and R (rotation matrix for each link).  The output will be in cell array.  It has 1  Nt (Nt: number of time discretization) cells.  In each time cell, there are another 1  Nb (Nb: number of branches) cells, and in each branch cell, another 1  Nj (Nj: number of joints) cells. Finally, each joint cells will have either 1  3 position vector or 3  3 link orientation matrix.

For easier output data access use:

- getPos(linkIdx, JK, Time)

- getAng(linkIdx, R, Time)

to get position vector and link orientation in quaternion angle. Positions vector will be output to [Nt by 3 (x, y, z) times Nj] matrix and the link orientation vector will be output to to [Nt by 4 (, x, y, z) times Nj].

The CoM of entire system is calculated using CenterOfMass.m.  It will output Nt by 3 (x, y, z) matrix.

Forward kinematics algorithm

Below shows the algorithm for the forward kinematics.

- child branches are sorted in the order of the least to the most amount of grandchild.  Similar is done for grand child branches.  This continues to the latest (youngest) branch.

- Forward kinematics (fk) of parent branch is calculated.  Then, fk of child 1 is calculated, grand child 1, and so on are calculated.  Then, the sister branches in that family are calculated

Examples human will make the algorithm more evident.

For human, fk is calculated in following manner:

- the fictitious branch

- the right leg and left leg branches

- the torso branch

- the neck, right arm and left arm branches

3. Visualization (stick figure)

Following the forward kinematics is visualization.

There are two important structures: Param and Anim.  Param allows users to change stick figure:

- fictitiousOn – turns on and off the fictitious branch

- fictitiousLine – defines fictitious branch line type and color (default: black dash line)

- is2D – used to add lateral part of body for sagittal plane (2D) models.  This is done by using anthropometry ratio.

- height – for the anthropometry ratio above

- CoMOn – turns on and off the CoM markers

- isStatic – determines whether static pose

- dhinfo – parent and child relationships

- parent – number of child a parent has

Anim is used to determine:

- speed: speed of animation

- plotFunc: function for drawing the system (e.g. biped stick figure, 3d bodies, drones, etc.); drawRobot is for drawing biped stick figure.  In order to animate other system, user need to make the function.

- verbose: allows user to control the animation during the play such as rolling backward, forward, etc.

There are three main functions for visualization:

- packFK.m: restructure forward kinematics structure and determines parent and child relationship.  The limitation of this code is that it:

  - assumes that system is a biped

  - does not have foot.

- drawRobot.m: draws a stick figure

- animateStickFig.m: animates the stick figure

The main purpose of packFK.m is to restructure forward kinematics output.  For better efficiency in a recursive loop, cell arrays are converted to double array.  A limitation of packFK.m is that it assumes that the system is a biped.  Thus, it would not work for any general structure.  A function that allows user to manually define order and how different branches are connected will resolve this issue.  Further detail is documented in the code.